https://randomnerdtutorials.com/esp32-freertos-arduino-tasks/

https://randomnerdtutorials.com/esp32-freertos-queues-inter-task-arduino/

ESP32 FreeRTOS:

FreeRTOS is a lightweight, open-source real-time operating system (RTOS).

It provides a framework for running multiple tasks concurrently,

each with its own priority and execution schedule. Instead of running code line by line,

FreeRTOS lets you create independent tasks that the ESP32 can switch between quickly

based on the task priority.

1) Creating Tasks:

Wire an LED to GPIO 2

code:

#define LED_PIN 2

// Declare task handle

TaskHandle_t BlinkTaskHandle = NULL;

//A TaskHandle_t is a variable that points to a FreeRTOS task, letting you control it,

like resuming it, stopping it, or delete it.

//This function is a FreeRTOS task, which is a special kind of function that runs

//independently under the FreeRTOS scheduler, allowing multitasking on the ESP32.

//FreeRTOS tasks must return void and must accept a single argument,

//which can be used to pass data to the function (not used in our case).

/*

vTaskDelay() is a FreeRTOS function that pauses a task for a specified number of ticks,

allowing other tasks to run during that time.

It doesn’t block your code like delay().

*/

/*

print in which core the task is running.

We can get that information by calling the xPortGetCoreID() function.

*/

void BlinkTask(void *parameter) {

for (;;) { // Infinite loop

digitalWrite(LED_PIN, HIGH);

Serial.println("BlinkTask: LED ON");

vTaskDelay(1000 / portTICK_PERIOD_MS); // 1000ms

digitalWrite(LED_PIN, LOW);

Serial.println("BlinkTask: LED OFF");

vTaskDelay(1000 / portTICK_PERIOD_MS);

Serial.print("BlinkTask running on core ");

Serial.println(xPortGetCoreID());

}

}

void setup() {

Serial.begin(115200);

pinMode(LED_PIN, OUTPUT);

/*

Now, to actually create a FreeRTOS task and assign it to a specific core,

we need to use the xTaskCreatePinnedToCore() function.

*/

xTaskCreatePinnedToCore(

BlinkTask,         // Task function

"BlinkTask",       // Task name

10000,             // Stack size (bytes)

NULL,              // Parameters

1,                 // Priority

&BlinkTaskHandle,  // Task handle

1                  // Core 1

);

}

void loop() {

// Empty because FreeRTOS scheduler runs the task

}

2)Suspend or resume task

We’ll create a task that blinks an LED, and use a pushbutton to control it.

When the button is pressed, the task will be suspended to stop the blinking,

and pressing it again will resume the task.

pushbutton to GPIO 23,LED to GPIO 2

code :

#define LED1_PIN 2

#define BUTTON_PIN 23

// Volatile variables for ISR

volatile bool taskSuspended = false;

volatile uint32_t lastInterruptTime = 0;

const uint32_t debounceDelay = 100;

/*To detect pushbutton presses, we’re using interrupts. When we use interrupts,

we need to define an interrupt service routine (a function that runs on the ESP32 RAM).

In this case, we create the buttonISR() function.

We must add IRAM_ATTR to the function definition to run it on RAM.*/

void IRAM_ATTR buttonISR() {

// Debounce

uint32_t currentTime = millis();

if (currentTime - lastInterruptTime < debounceDelay) {

return;

}

lastInterruptTime = currentTime;

// Toggle task state

taskSuspended = !taskSuspended;

if (taskSuspended) {

vTaskSuspend(BlinkTaskHandle);

Serial.println("BlinkTask Suspended");

} else {

vTaskResume(BlinkTaskHandle);

Serial.println("BlinkTask Resumed");

}

}

void BlinkTask(void *parameter) {

for (;;) { // Infinite loop

digitalWrite(LED1_PIN, HIGH);

Serial.println("BlinkTask: LED ON");

vTaskDelay(1000 / portTICK_PERIOD_MS); // 1000ms

digitalWrite(LED1_PIN, LOW);

Serial.println("BlinkTask: LED OFF");

Serial.print("BlinkTask running on core ");

Serial.println(xPortGetCoreID());

vTaskDelay(1000 / portTICK_PERIOD_MS);

}

}

void setup() {

Serial.begin(115200);

// Initialize pins

pinMode(LED1_PIN, OUTPUT);

pinMode(BUTTON_PIN, INPUT_PULLUP); // Internal pull-up resistor

/*

To set an interrupt in the Arduino IDE, you use the attachInterrupt() function,

that accepts as arguments:

the GPIO pin, the name of the function to be executed, and mode

*/

// Attach interrupt to button

attachInterrupt(digitalPinToInterrupt(BUTTON_PIN), buttonISR, FALLING);

// Create task

xTaskCreatePinnedToCore(

BlinkTask,         // Task function

"BlinkTask",       // Task name

10000,             // Stack size (bytes)

NULL,              // Parameters

1,                 // Priority

&BlinkTaskHandle,  // Task handle

1                  // Core 1

);

}

void loop() {

// Empty because FreeRTOS scheduler runs the task

}

3) Creating and Running Multiple Tasks

#define LED1_PIN 2

#define LED2_PIN 4

TaskHandle_t Task1Handle = NULL;

TaskHandle_t Task2Handle = NULL;

void Task1(void *parameter) {

pinMode(LED1_PIN, OUTPUT);

for (;;) {

digitalWrite(LED1_PIN, HIGH);

Serial.println("Task1: LED1 ON");

vTaskDelay(1000 / portTICK_PERIOD_MS);

digitalWrite(LED1_PIN, LOW);

Serial.println("Task1: LED1 OFF");

vTaskDelay(1000 / portTICK_PERIOD_MS);

}

}

void Task2(void *parameter) {

pinMode(LED2_PIN, OUTPUT);

for (;;) {

digitalWrite(LED2_PIN, HIGH);

Serial.println("Task2: LED2 ON");

vTaskDelay(333 / portTICK_PERIOD_MS);

digitalWrite(LED2_PIN, LOW);

Serial.println("Task2: LED2 OFF");

vTaskDelay(333 / portTICK_PERIOD_MS);

}

}

void setup() {

Serial.begin(115200);

xTaskCreatePinnedToCore(

Task1,             // Task function

"Task1",           // Task name

10000,             // Stack size (bytes)

NULL,              // Parameters

1,                 // Priority

&Task1Handle,      // Task handle

1                  // Core 1

);

xTaskCreatePinnedToCore(

Task2,            // Task function

"Task2",          // Task name

10000,            // Stack size (bytes)

NULL,             // Parameters

1,                // Priority

&Task2Handle,     // Task handle

1                 // Core 1

);

}

void loop() {

// Empty because FreeRTOS scheduler runs the task

}

4) Creating and Running Multiple Tasks On Different Cores

//The only difference in this example is that we define Task2 to run on core 0 as shown below.

#define LED1_PIN 2

#define LED2_PIN 4

TaskHandle_t Task1Handle = NULL;

TaskHandle_t Task2Handle = NULL;

void Task1(void *parameter) {

pinMode(LED1_PIN, OUTPUT);

for (;;) {

digitalWrite(LED1_PIN, HIGH);

Serial.println("Task1: LED1 ON");

vTaskDelay(1000 / portTICK_PERIOD_MS);

digitalWrite(LED1_PIN, LOW);

Serial.println("Task1: LED1 OFF");

vTaskDelay(1000 / portTICK_PERIOD_MS);

Serial.print("Task 1 running on core ");

Serial.println(xPortGetCoreID());

}

}

void Task2(void *parameter) {

pinMode(LED2_PIN, OUTPUT);

for (;;) {

digitalWrite(LED2_PIN, HIGH);

Serial.println("Task2: LED2 ON");

vTaskDelay(333 / portTICK_PERIOD_MS);

digitalWrite(LED2_PIN, LOW);

Serial.println("Task2: LED2 OFF");

vTaskDelay(333 / portTICK_PERIOD_MS);

Serial.print("Task 2 running on core ");

Serial.println(xPortGetCoreID());

}

}

void setup() {

Serial.begin(115200);

xTaskCreatePinnedToCore(

Task1,             // Task function

"Task1",           // Task name

10000,             // Stack size (bytes)

NULL,              // Parameters

1,                 // Priority

&Task1Handle,      // Task handle

1                  // Core 1

);

xTaskCreatePinnedToCore(

Task2,            // Task function

"Task2",          // Task name

10000,            // Stack size (bytes)

NULL,             // Parameters

1,                // Priority

&Task2Handle,     // Task handle

0                 // Core 0

);

}

void loop() {

// Empty because FreeRTOS scheduler runs the task

}

5)stack and heap usage :

stack Usage:

The stack is a dedicated memory area for each FreeRTOS task,

used to store temporary data like local variables, function call information,

and task state during execution.

Each task has its own stack, allocated when the task is created.

here is a function that you can call inside your task to determine the stack usage:

the uxTaskGetStackHighWaterMark() function

Heap Usage:

The heap is a shared memory pool in the ESP32’s SRAM,

used for dynamic memory allocation, including task stacks, buffers,

and other runtime data allocated by FreeRTOS or the Arduino core.

#define LED1_PIN 2

#define LED2_PIN 4

TaskHandle_t Task1Handle = NULL;

TaskHandle_t Task2Handle = NULL;

void Task1(void *parameter) {

pinMode(LED1_PIN, OUTPUT);

for (;;) {

digitalWrite(LED1_PIN, HIGH);

Serial.println("Task1: LED1 ON");

vTaskDelay(1000 / portTICK_PERIOD_MS);

digitalWrite(LED1_PIN, LOW);

Serial.println("Task1: LED1 OFF");

vTaskDelay(1000 / portTICK_PERIOD_MS);

Serial.printf("Task1 Stack Free: %u bytes\n", uxTaskGetStackHighWaterMark(NULL));

}

}

void Task2(void *parameter) {

pinMode(LED2_PIN, OUTPUT);

for (;;) {

digitalWrite(LED2_PIN, HIGH);

Serial.println("Task2: LED2 ON");

vTaskDelay(333 / portTICK_PERIOD_MS);

digitalWrite(LED2_PIN, LOW);

Serial.println("Task2: LED2 OFF");

vTaskDelay(333 / portTICK_PERIOD_MS);

Serial.printf("Task2 Stack Free: %u bytes\n", uxTaskGetStackHighWaterMark(NULL));

}

}

void setup() {

Serial.begin(115200);

delay(1000);

Serial.printf("Starting FreeRTOS: Memory Usage\nInitial Free Heap: %u bytes\n", xPortGetFreeHeapSize());

xTaskCreatePinnedToCore(

Task1,

"Task1",

10000,

NULL,

1,

&Task1Handle,

1

);

xTaskCreatePinnedToCore(

Task2,

"Task2",

10000,

NULL,

1,

&Task2Handle,

1

);

}

void loop() {

static uint32_t lastCheck = 0;

if (millis() - lastCheck > 5000) {

Serial.printf("Free Heap: %u bytes\n", xPortGetFreeHeapSize());

lastCheck = millis();

}

}

ESP32 with FreeRTOS Queues: Inter-Task Communication

- Queues allow you to exchange data between tasks

- Queues: allow tasks to send and receive data, like sensor readings, button states, or other data

- Semaphores are like signaling tools in FreeRTOS used to coordinate tasks. They are often used to indicate that an event has occurred or a resource is ready. They don’t carry data, just a count (or flag) to trigger actions after something has happened.

- Mutexes (mutual exclusions) are used to protect shared resources by allowing only one task to access them at a time. This prevents conflicts in concurrent operations that need to access the same data

FreeRTOS queue:

1)creating a queue:

xQueueCreate(size, item_size)

size: no of items,

item_size:  the bytes size of heap allocated for each item on the queue.

2)Sending data to queue:

To send data to a queue use the xQueueSend() to add data to the queue. Or use xQueueSendFromISR() if sending the data from an ISR (interrupt service routine).

3)Receive data from queue:

The xQueueReceive() function reads data from the queue, if available.

Code : control LED with potentiometer

1)Define the size of the FreeRTOS queue.

2)create a queue handle

3) Create a function for the potentiometer task: potTask

4) reate the function for the task that controls the LED brightness.

5) send item to queue using xQueueSend()  here we are sending potvalue

6) To get values from the queue we can use the xQueueReceive() function.

Pass the queue handle as an argument, a pointer to the variable where the value should be saved and whether to wait for an item to be available (portMAX_DELAY) or to return immediately (pass 0)

7) Set up the PWM properties.

ledcAttach(LED_PIN, PWM_FREQ, PWM_RESOLUTION);

wire an LED and a potentiometer to your ESP32 board. We’re connecting the LED to GPIO 2 and the potentiometer to GPIO 15.

#define POT_PIN 15

#define LED_PIN 2

#define PWM_FREQ 5000

#define PWM_RESOLUTION 12  // 12-bit (0–4095)

#define QUEUE_SIZE 5

8) create queue then create task

// Create an handle for the queue

QueueHandle_t potQueue = NULL;

void potTask(void *parameter) {

for (;;) {

uint16_t potValue = analogRead(POT_PIN);  // Read 0–4095

xQueueSend(potQueue, &potValue, portMAX_DELAY);  // Send to queue

Serial.printf("potTask: Sent pot value %u\n", potValue);

vTaskDelay(100 / portTICK_PERIOD_MS);  // 100ms

}

}

void LEDBrightnessTask(void *parameter) {

for (;;) {

uint16_t potValue;

if (xQueueReceive(potQueue, &potValue, portMAX_DELAY)) {

uint16_t brightness = potValue;

ledcWrite(LED_PIN, brightness);

Serial.printf("LEDBrightnessTask: Set brightness %u\n", brightness);

}

}

}

void setup() {

Serial.begin(115200);

// Setup PWM for LED

ledcAttach(LED_PIN, PWM_FREQ, PWM_RESOLUTION);

// Create queue (5 items, each uint16_t)

potQueue = xQueueCreate(QUEUE_SIZE, sizeof(uint16_t));

if (potQueue == NULL) {

Serial.println("Failed to create queue!");

while (1);

}

// Create tasks

xTaskCreatePinnedToCore(

potTask,

"potTask",

3000,  // Task stack

NULL,

1,

NULL,

1  // Core 1

);

xTaskCreatePinnedToCore(

LEDBrightnessTask,

"LEDBrightnessTask",

3000,

NULL,

1,

NULL,

);

}

void loop() {

// Empty

}

Semaphore:

used to indicate that an event has occurred or that a resource is available.

Semaphore carry count or flag or signal used to trigger actions when something happens.

They allow tasks to wait for events, such as a button press or motion detection, or to signal that an event has occurred

There are two types of semaphores:

- Binary semaphore: signals a single event. It is a synchronization tool that can be either empty (0) or full (1). It’s like a signal that a task is waiting for before it can proceed.

- Counting semaphore: tracks multiple events (it can be the same event several times). It’s like a queue of events up to a maximum count that you define. Contrary to FreeRTOS queues, these don’t carry data. Only a synchronization signal. You’ll better understand how this works later in the example.

Create a Binary semaphore:

To create a binary semaphore, use the xSemaphoreCreateBinary() function. It returns a SemaphoreHandle_t handle if successful, or NULL if the creation fails.

Create a counting semaphore:

To create a counting semaphore, use the xSemaphoreCreateCounting() function. It returns a SemaphoreHandle_t handle if successful, or NULL if the creation fails. Pass as an argument the maximum count.

Taking a semaphore or getting form semaphore:

The task is trying to acquire (lock) the semaphore

For example, before reading from a shared sensor, a task might take the semaphore to make sure no other task is using the sensor.

Use the xSemaphoreTake(semaphore, timeout) function in a task to wait for or take a semaphore. For a binary semaphore, it blocks until the semaphore is available (state 1), setting it to 0 when taken.

For a counting semaphore, it decrements the count if greater than 0, or blocks if the count is 0. The timeout parameter specifies how long to wait (in ticks); portMAX_DELAY means wait indefinitely. This means the task will be blocked until there’s a semaphore value to take.

### Giving a Semaphore: The task is releasing (unlocking) or signaling the semaphore.

To give a semaphore use the xSemaphoreGive() function if inside a task, or xSemaphoreGiveFromISR() if used in ISRs (interrupt service routine functions).

For a binary semaphore, it sets the state to 1, unblocking a waiting task (or ignored if the semaphore is already 1). For a counting semaphore, it increments the count up to maxCount, unblocking a waiting task (ignored if at maxCount).

Example 1:

- Red LED connected to GPIO 2

- Blue LED connected to GPIO 4

- Pushbutton connected to GPIO 23

Steps:

- Create an handle for the semaphore called buttonSemaphore

- create a binary semaphore using the xSemaphoreCreateBinary()

- create the LedToggleTask and the LEDBlinkTask with different priorities.

- When the button is pressed, the buttonISR() function will run. If we have a valid press, it uses FreeRTOS semaphore functions to signal another task that the button event has occurred.

##### 5) higherPriorityTaskWoken and portYIELD_FROM_ISR

The higherPriorityTaskWoken variable is used to check if giving the semaphore will wake up a task that has a higher priority than the currently running task. If it does, we call portYIELD_FROM_ISR() to let the system immediately switch to that higher-priority task right after the interrupt finishes. In our case, we want to switch immediately to the LEDToggleTask.

In other words:

- Basically, higherPriorityTaskWoken is used to check if giving the semaphore unblocked a more important task.

- We pass it to xSemaphoreGiveFromISR() so it can update the value.

- If it’s pdTRUE, we call portYIELD_FROM_ISR() to let FreeRTOS switch to that task immediately.

6)

#define BUTTON_PIN 23

#define LED1_PIN 2   // Toggled LED

#define LED2_PIN 4   // Blinking LED

#define DEBOUNCE_DELAY 200

SemaphoreHandle_t buttonSemaphore = NULL;

volatile uint32_t lastInterruptTime = 0;

void IRAM_ATTR buttonISR() {

uint32_t currentTime = millis();

if (currentTime - lastInterruptTime > DEBOUNCE_DELAY) {

BaseType_t higherPriorityTaskWoken = pdFALSE;

xSemaphoreGiveFromISR(buttonSemaphore, &higherPriorityTaskWoken);

lastInterruptTime = currentTime;

if (higherPriorityTaskWoken) {

portYIELD_FROM_ISR();

}

}

}

void LEDToggleTask(void *parameter) {

pinMode(LED1_PIN, OUTPUT);

bool ledState = false;

for (;;) {

if (xSemaphoreTake(buttonSemaphore, portMAX_DELAY)) {

ledState = !ledState;

digitalWrite(LED1_PIN, ledState ? HIGH : LOW);

Serial.print("LEDToggleTask: LED1 ");

Serial.println(ledState ? "ON" : "OFF");

}

}

}

void LEDBlinkTask(void *parameter) {

pinMode(LED2_PIN, OUTPUT);

for (;;) {

digitalWrite(LED2_PIN, HIGH);

Serial.println("LEDBlinkTask: LED2 ON");

vTaskDelay(250 / portTICK_PERIOD_MS);

digitalWrite(LED2_PIN, LOW);

Serial.println("LEDBlinkTask: LED2 OFF");

vTaskDelay(250 / portTICK_PERIOD_MS);

}

}

void setup() {

Serial.begin(115200);

// Defining the button as an interrupt

pinMode(BUTTON_PIN, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(BUTTON_PIN), buttonISR, FALLING);

buttonSemaphore = xSemaphoreCreateBinary();

if (buttonSemaphore == NULL) {

Serial.println("Failed to create semaphore!");

while (1);

}

xTaskCreatePinnedToCore(

LEDToggleTask,          // Task function

"LEDToggleTask",        // Task name

3000,                   // Stack size

NULL,                   // Task parameters

2,                      // Higher priority

NULL,                   // Task handle

1                       // Core ID

);

xTaskCreatePinnedToCore(

LEDBlinkTask,           // Task function

"LEDBlinkTask",         // Task name

3000,                   // Stack size

NULL,                   // Task parameters

1,                      // Medium priority

NULL,                   // Task handle

1                       // Core ID

);

}

void loop() {

}